Part A: Circuit Calculations

# Circuit (a): Series (R1 = 20 Ω, R2 = 30 Ω, R3 = 50 Ω)

### Step 1: Total resistance

R_T = R1 + R2 + R3

R_T = 20 + 30 + 50 = 100 Ω

### Step 2: Total current

I_T = V_S / R_T = 125 / 100 = 1.25 A

### Step 3: Current through each resistor

A series circuit has one path, so by Kirchhoff's Current Law the same current flows through every resistor:

I1 = I2 = I3 = I_T = 1.25 A

### Step 4: Voltage drop across each resistor (V = I × R)

V1 = 1.25 × 20 = 25 V

V2 = 1.25 × 30 = 37.5 V

V3 = 1.25 × 50 = 62.5 V

Check (Kirchhoff's Voltage Law): 25 + 37.5 + 62.5 = 125 V = V_S.

### Step 5: Power dissipated (P = V × I)

P1 = 25 × 1.25 = 31.25 W

P2 = 37.5 × 1.25 = 46.875 W

P3 = 62.5 × 1.25 = 78.125 W

Cross-check with P = I²R: 1.5625 × 20 = 31.25 W; 1.5625 × 30 = 46.875 W; 1.5625 × 50 = 78.125 W. Total P = 156.25 W, and the source supplies 125 × 1.25 = 156.25 W, so power is conserved.

### Summary (a)

| Resistor | R (Ω) | I (A) | V (V) | P (W) |
| --- | --- | --- | --- | --- |
| R1 | 20 | 1.25 | 25.0 | 31.25 |
| R2 | 30 | 1.25 | 37.5 | 46.875 |
| R3 | 50 | 1.25 | 62.5 | 78.125 |
| Total | 100 | 1.25 | 125.0 | 156.25 |

# Circuit (b): Parallel

### Step 1: Total resistance

1/R_T = 1/R1 + 1/R2 + 1/R3

1/R_T = 1/20 + 1/100 + 1/50 = 0.05 + 0.01 + 0.02 = 0.08 S

R_T = 1 / 0.08 = 12.5 Ω

Cross-check: R1 ∥ R3 = (20 × 50)/(20 + 50) = 14.286 Ω; then 14.286 ∥ 100 = 1428.6/114.286 = 12.5 Ω.

### Step 2: Total current

I_T = V_S / R_T = 125 / 12.5 = 10 A

### Step 3: Current through each resistor

Every branch is connected directly across the source, so each has 125 V across it. Applying Ohm's Law to each branch:

I1 = 125 / 20 = 6.25 A

I2 = 125 / 100 = 1.25 A

I3 = 125 / 50 = 2.5 A

Check (Kirchhoff's Current Law): 6.25 + 1.25 + 2.5 = 10 A = I_T.

### Step 4: Voltage drop across each resistor

V1 = V2 = V3 = 125 V

### Step 5: Power dissipated (P = V × I)

P1 = 125 × 6.25 = 781.25 W

P2 = 125 × 1.25 = 156.25 W

P3 = 125 × 2.5 = 312.5 W

Cross-check with P = V²/R: 15625/20 = 781.25 W; 15625/100 = 156.25 W; 15625/50 = 312.5 W. Total P = 1250 W = 125 × 10 = P_S.

### Summary (b)

| Resistor | R (Ω) | V (V) | I (A) | P (W) |
| --- | --- | --- | --- | --- |
| R1 | 20 | 125 | 6.25 | 781.25 |
| R2 | 100 | 125 | 1.25 | 156.25 |
| R3 | 50 | 125 | 2.50 | 312.50 |
| Total | 12.5 | 125 | 10.00 | 1250 |

- Series: current is common and voltage divides in proportion to resistance, so the largest resistor drops the most voltage and dissipates the most power.

- Parallel: voltage is common and current divides inversely to resistance, so the smallest resistor draws the most current and dissipates the most power.

- With the same supply, the parallel circuit draws 8 times the current (10 A vs 1.25 A) and dissipates 8 times the power (1250 W vs 156.25 W).

- These power levels far exceed ordinary resistor ratings (0.25 W to a few watts), so a physical build would need high-wattage power resistors. The schematics list the minimum rating needed per component.