Circuit-Design summary

# Contents

| File | Description |
| --- | --- |
| circuit_a_series.pdf | Part B1: series circuit schematic with component table |
| circuit_b_parallel.pdf | Part B1: parallel circuit schematic with component table |
| esp32_dht22.pdf | Part B2: ESP32 + DHT22 schematic with component table |

# Part A summary (125 V source)

| Circuit | R_T | I_T | Current per resistor | Voltage per resistor | Power per resistor |
| --- | --- | --- | --- | --- | --- |
| (a) Series 20/30/50 Ω | 100 Ω | 1.25 A | 1.25 A each | 25 / 37.5 / 62.5 V | 31.25 / 46.875 / 78.125 W (total 156.25 W) |
| (b) Parallel 20/100/50 Ω | 12.5 Ω | 10 A | 6.25 / 1.25 / 2.5 A | 125 V each | 781.25 / 156.25 / 312.5 W (total 1250 W) |

# Component tables

## Circuit (a): series

| Ref | Component | Value | Qty | Rating needed |
| --- | --- | --- | --- | --- |
| V1 | DC voltage source | 125 V | 1 | ≥ 1.25 A |
| R1 | Resistor | 20 Ω | 1 | ≥ 31.25 W |
| R2 | Resistor | 30 Ω | 1 | ≥ 46.875 W |
| R3 | Resistor | 50 Ω | 1 | ≥ 78.125 W |

## Circuit (b): parallel

| Ref | Component | Value | Qty | Rating needed |
| --- | --- | --- | --- | --- |
| V1 | DC voltage source | 125 V | 1 | ≥ 10 A |
| R1 | Resistor | 20 Ω | 1 | ≥ 781.25 W |
| R2 | Resistor | 100 Ω | 1 | ≥ 156.25 W |
| R3 | Resistor | 50 Ω | 1 | ≥ 312.5 W |

## ESP32 + DHT22

| Ref | Component | Value / Part | Qty | Purpose |
| --- | --- | --- | --- | --- |
| U1 | ESP32 module | ESP32-WROOM-32E | 1 | Microcontroller |
| U2 | LDO regulator | AMS1117-3.3 | 1 | 5 V to 3.3 V |
| U3 | Sensor | DHT22 (AM2302) | 1 | Temperature/humidity |
| R1 | Resistor | 10k | 1 | DATA pull-up |
| R2 | Resistor | 10k | 1 | EN pull-up |
| R3 | Resistor | 10k | 1 | GPIO0 pull-up |
| C1 | Capacitor | 10uF | 1 | Regulator input |
| C2 | Capacitor | 22uF | 1 | Regulator output |
| C3 | Capacitor | 0.1uF | 1 | ESP32 decoupling |
| C4 | Capacitor | 0.1uF | 1 | DHT22 decoupling |
| C5 | Capacitor | 1uF | 1 | EN reset delay |
| SW1 | Tactile switch | RESET | 1 | Reset |
| SW2 | Tactile switch | BOOT | 1 | Flash mode (GPIO0 low) |
| J1 | Header 1×2 | 5V IN | 1 | Power input |
| J2 | Header 1×3 | SERIAL | 1 | GND/TX/RX programming |

# ESP32 + DHT22 design notes

- 5 V input (J1) is regulated to 3.3 V by U2. The ESP32 and DHT22 both run from 3.3 V (the DHT22 accepts 3.3–5 V), so the DATA line is safe for the ESP32 GPIO.

- DHT22 DATA connects to GPIO4 (IO4) with a 10k pull-up (R1) to 3.3 V. The DHT22 symbol labels this pin SDA, but the sensor uses a single-wire protocol, not I²C. The SCK pin is not connected (no-connect flag).

- EN has a 10k pull-up (R2), a 1uF delay capacitor (C5) and a RESET switch (SW1). GPIO0 has a 10k pull-up (R3) and a BOOT switch (SW2) to enter flash mode.

- J2 provides GND, TX (GPIO1/TXD0) and RX (GPIO3/RXD0) for serial programming.

- Nets are connected with net labels and GND symbols: +5V, +3V3, GND, EN, GPIO0, DHT_DATA, UART_TX, UART_RX.

# Note on reference designators

EasyEDA Standard treats all sheets in one project as a single schematic, so it warns about duplicate prefixes (V1 and R1–R3 appear on more than one sheet). Each sheet is an independent circuit with its own component table, so the repeated references are intentional.